Getting the New Software – Updated 4/10/25

Software Download Link for GVC1 Prime 9383 (Blue control board):

https://drive.google.com/file/d/1iq10RWywA5icQqFrHoeTnvCCUNpLQbiy/view?usp=sharing

Software Download Link for GVC2 Prime 13150 (Red control board):

https://drive.google.com/file/d/1eXj8UjMnhrVFDIq5-GFnXuu7AjK-bUiU/view?usp=sharing

| - From the download links above choose either GVC1 Prime or GVC2 Prime based on your control board type. - Download and save the file either to your Desktop or to your Downloads folder. |  |
| --- | --- |
| - Locate the downloaded file |  |
| - Rename the file to extension from “.bi_” to “.bin” |  |
| - Save the file to a USB drive - The file must Not be in a folder. (the root) |  |

Programming the control board

Parts Needed

- USB Drive with software

- Jumper or coin to short programming pins

| - Using the machines power switch turn the power off - Locate USB port in the center of the control board |  |
| --- | --- |
| - Insert USB Drive in Port |  |
| - Locate Programming Pins next to USB port. |  |
| - Use jumper or a coin to short the Programming Pins together. |  |
| - With the jumper (or coin) still on the programming pins turn the power on to the machine. - In the upper left-hand corner of the control board a light will rapidly flash while the board is programming. - If the light flashes slowly, your USB drive is not compatible. Try a different USB drive. - When the light has stopped flashing, programming is complete, turn the machine power off - Remove USB drive and jumper or coin. - Turn power on - NOTE: Once you have programmed the control board, all setting returned to the factory default settings. - Be sure to do a motor count, set prices and any other settings your machine may need such as, refrigeration, space to sales, etc. |  |

Link to video of programming process:

https://www.dropbox.com/s/zpfx9v888rz10yv/20210128_165703000_iOS.mp4?dl=0